# Cisco Packet Tracer — Command Sheet

Start every block with:  enable  →  configure terminal

# A. Switching

## 1. VLAN + access port   Switch

vlan 10

name CSE

interface fa0/1

switchport mode access

switchport access vlan 10

Verify: show vlan brief

## 2. Trunk + native VLAN   Switch (both ends)

interface gi0/1

switchport trunk encapsulation dot1q   ! 3560 only

switchport mode trunk

switchport trunk native vlan 99

Verify: show interfaces trunk

## 3. Router-on-a-Stick   Router

interface gi0/0

no shutdown

interface gi0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface gi0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

Verify: show ip interface brief

## 4. Multilayer switch (SVI)   3560 switch

ip routing

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

Verify: show ip route

# B. Addressing and services

## 5. VLSM   On paper

Biggest network first  ·  hosts = 2^h − 2  ·  nth usable = network + n  (gateway 1st, server 2nd, first PC 10th)

## 6. DHCP on a router   Router

ip dhcp excluded-address 192.168.10.1 192.168.10.9

ip dhcp pool CSE

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 192.168.50.3

Verify: show ip dhcp binding

## 7. DHCP relay   PCs' gateway interface

interface gi0/0.10

ip helper-address 192.168.50.4

Verify: ipconfig   (on the PC)

## 8. DHCP server   Packet Tracer Server GUI

Server → Services → DHCP → On → Default Gateway, DNS Server, Start IP, Subnet Mask, Max Users → Add

## 9. DNS   Server GUI + router

Server → Services → DNS → On → Name www.kuet.ac.bd, Type A Record, Address 203.0.113.5 → Add

no ip domain-lookup

Verify: nslookup www.kuet.ac.bd   (on the PC)

# C. Routing

## 10. Static + default route   Router

ip route 192.168.2.0 255.255.255.0 10.0.0.2

ip route 0.0.0.0 0.0.0.0 200.1.1.2

Verify: show ip route

## 11. OSPF   Every router

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

default-information originate   ! ISP-facing router only

Verify: show ip ospf neighbor

## 12. RIPv2   Every router

router rip

version 2

no auto-summary

network 192.168.1.0

network 10.0.0.0

Verify: show ip route

## 13. GRE tunnel   R1 (R2 mirror)

ip route 0.0.0.0 0.0.0.0 200.1.1.2

interface tunnel 0

ip address 172.16.0.1 255.255.255.252

tunnel source gi0/1

tunnel destination 200.1.2.1

router rip

version 2

no auto-summary

network 172.16.0.0

network 192.168.10.0

Verify: ping 172.16.0.2

R2: tunnel IP 172.16.0.2, tunnel destination 200.1.1.1, its own LAN in RIP.

# D. NAT

## 14. NAT inside / outside   Edge router

interface gi0/0

ip nat inside

interface gi0/1

ip nat outside

## 15. Static NAT + port forwarding   Edge router

ip nat inside source static 192.168.50.2 203.0.113.6

ip nat inside source static tcp 192.168.50.2 80 203.0.113.5 80

ip nat inside source static udp 192.168.50.3 53 203.0.113.5 53

Verify: show ip nat translations

## 16. PAT (overload)   Edge router

access-list 1 permit 192.168.10.0 0.0.0.255

ip nat inside source list 1 interface gi0/1 overload

! or with a pool:

ip nat pool KUET 203.0.113.20 203.0.113.30 netmask 255.255.255.0

ip nat inside source list 1 pool KUET overload

Verify: show ip nat translations

# E. Security and VPN

## 17. Basic device security   Every router / switch

enable secret Admin123

line console 0

password Con123

login

line vty 0 4

password Vty123

login

service password-encryption

banner motd #Authorized access only#

Verify: show running-config

## 18. SSH   Router / switch

hostname R1

ip domain-name kuet.ac.bd

username admin secret Admin123

crypto key generate rsa   ! modulus: 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

Verify: ssh -l admin 192.168.10.1   (from the PC)

## 19. Password recovery   Router, console cable

! power on, press Ctrl+Break

confreg 0x2142

reset

! setup dialog: no

enable

copy startup-config running-config

configure terminal

no enable secret

enable secret NewPass123

interface gi0/0

no shutdown

exit

config-register 0x2102

end

copy running-config startup-config

Verify: show version

## 20. GRE over IPsec VPN   R1 (R2 mirror)

crypto isakmp policy 10

encryption 3des

hash md5

authentication pre-share

group 5

crypto isakmp key VPNkey123 address 200.1.2.1

crypto ipsec transform-set MYSET esp-3des esp-md5-hmac

access-list 110 permit gre host 200.1.1.1 host 200.1.2.1

crypto map MYMAP 10 ipsec-isakmp

set peer 200.1.2.1

set transform-set MYSET

match address 110

interface gi0/1

crypto map MYMAP

Verify: show crypto isakmp sa   ·   show crypto ipsec sa

R2: same commands with 200.1.1.1 and 200.1.2.1 swapped.

# Verify commands

show ip interface brief

show vlan brief

show interfaces trunk

show ip route

show ip ospf neighbor

show ip dhcp binding

show ip nat translations

show crypto isakmp sa

show crypto ipsec sa